e/;izns’k xzkeh.k lM+d fodkl izkf/kdj.k

¼e-iz- 'kklu iapk;r ,oa xzkeh.k fodkl foHkkx ds v/khu½

[k.M&2 iape ry] i;kZokl Hkou]vjsjk fgYl Hkksiky

Ø- 4514 @22@fo&12@xzk-l-izk@Vh&7@2012                     Hkksiky] fnukad 30 @03@2012

la'kks/ku vkns'k

dk;kZy;hu vkns'k Ø-15741 @22@fo&12@xzk-l-izk@Vh&7@2011 Hkksiky] fnukad   08@11@2011 }kjk eq[;ea=h xzke lM+d ;kstuk ds dk;Z  gsrq izkf/kdj.k ds ,l-vks-vkj ¼fnukad 15-06-2011½ esa xzsoy csldkslZ ,oa ljQsl dkslZ ds fuEukafdr vk;Ve xzkeh.k ;kaf=dh lsok ds ,l-vks-vkj- ¼fodkl vk;qDr ds i= Ø- 4742@1398@fo&10@vkjlh@xzk;kals Hkksiky fnukad 02-08-2010½ ls m)fjr dj izkf/kdj.k ds varxZr eq[;ea=h xzke lM+d ;kstuk ds dk;ksZ gsrq ykxw fd;s x;s FksA mDr vk;VEl ds ukseuDyspj esa mPp Lrjh; lfefr }kjk fnukad 29-03-2012 dks vkaf'kd la'kks/ku fd;k x;k gSA vr% mDr ifjisz{; esa bl dk;kZy; }kjk tkjh fd;s x;s mijksDr vkns'k esa rnkuqlkj vkaf'kd la'kks/ku fd;k tkrk gSA vr% vk;VEl dk fooj.k fuEukuqlkj i<+k tkosa %&

| Sl. No. | MPRRDA SOR ITEM No. | Item | Unit | Rates |
| --- | --- | --- | --- | --- |
| 1 | Gravel-1 | Construction of Gravel Base Course as per clause 402 of "Ministry of Rural Development, Specifications for Rural Roads-August, 2004" by an appropriate combination of natural gravel, river borne material and crushed metal in required proportions confirming to grading requirements specified in clause 2.2.2.1 of IRC :SP: 77-2008, having Liquid Limit and Plasticity Index less than 25 and 10 respectively, including all leads and lifts of material etc. complete. The mixing shall be by Mix-in–Place method over prepared surface, spreading in uniform layers with tractor mounted grader or manually, watering at OMC and compaction by smooth wheeled/vibratory roller of 80 to 100 kN static weight to achieve 100% of MDD as per IS:2720 (Part-7)".Above item having material strength not less than 30% CBR. | Cum | 447.90 |
| 2 | Gravel-2 | Construction of Gravel Surface Course as per clause 402 of "Ministry of Rural Development, Specifications for Rural Roads-August, 2004" by an appropriate combination of natural gravel, river borne material and crushed metal in required proportions confirming to grading requirements specified in clause 2.2.21 of IRC :SP: 77-2008, having Liquid Limit less than 40 and Plasticity Index 6-15 and strength of material not less than 20% CBR including all leads and lifts of material etc. complete. The mixing shall be by Mix-in–Place method over prepared surface, spreading in uniform layers with tractor mounted grader or manually, watering at OMC and compaction by smooth wheeled/vidratory roller of 80 to 100 kN static weight to achieve 100% of MDD as per IS:2720 (Part-7)". | Cum | 419.80 |
| 3 | Gravel-3 | Construction of Gravel Base Course as per clause 402 of "Ministry of Rural Development, Specifications for Rural Roads-August, 2004" by an appropriate combination of natural gravel, river borne material and crushed metal in required proportions confirming to grading requirements specified in clause 2.2.2.2 or 2.2.2.3 (Table 2.3, Material having Nominal Maximum size 80mm or 40 mm or 20 mm) of IRC :SP: 77-2008, having Liquid Limit and Plasticity Index less than 25 and 10 respectively, including all leads and lifts of material etc. complete. The mixing shall be by Mix-in–Place method over prepared surface, spreading in uniform layers with tractor mounted grader or manually, watering at OMC and compaction by smooth wheeled/vibratory roller of 80 to 100 kN static weight to achieve 100% of MDD as per IS:2720 (Part-7)".Above item having material strength not less than 30% CBR. | Cum | 427.70 |
| 4 | Gravel-4 | Construction of Gravel Surface Course as per clause 402 of "Ministry of Rural Development, Specifications for Rural Roads-August, 2004" by an appropriate combination of natural gravel, river borne material and crushed metal in required proportions confirming to grading requirements specified in clause 2.2.2.2 or 2.2.2.3 (Table 2.3, Material having Nominal Maximum size 20 mm or 10 mm) of IRC :SP: 77-2008, having Liquid Limit less than 40 and Plasticity Index 6-15 and strength of material not less than 20% CBR, including all leads and lifts of material etc. complete. The mixing shall be by Mix-in–Place method over prepared surface, spreading in uniform layers with tractor mounted grader or manually, watering at OMC and compaction by smooth wheeled/vibratory roller of 80 to 100 kN static weight to achieve 100% of MDD as per IS:2720 (Part-7)". | Cum | 360.40 |

mijksDr vk;Ve eq[;ea=h lM+d ;kstuk gsrq rRdky izHkko ls ykxw gksxsA

¼,e-ds-xqIrk½

izeq[k vfHk;ark

e-iz-xzkeh.k lM+d fodkl izkf/kdj.k

Hkksiky

i`-Ø- 4515 @22@fo&12@xzk-l-izk@Vh&7@2012            Hkksiky] fnukad    30@03@2012

izfrfyfi%&

- eq[; egkizca/kd I,II,III,IV e-iz-xzkeh.k lM+d fodkl izkf/kdj.k Hkksiky@bankSj@tcyiqj@jhok dh vksj lwpukFkZ ,oa vko’;d dk;Zogh gsrqA

- eq[; vfHk;ark iwoZ ifj{ks=@if'pe ifj{ks= xzkeh.k ;kaf=dh lsok HkksikyA

- eq[; egkizca/kd ¼foRr½ e-iz-xzkeh.k lM+d fodkl izkf/kdj.k dh vksj lwpukFkZ ,oa vko’;d dk;Zogh gsrqA

- egkizca/kd ¼leLr½ e-iz-xzkeh.k lM+d fodkl izkf/kdj.k dh vksj lwpukFkZ ,oa vko’;d dk;Zogh gsrqA

- egkizca/kd ¼rd½ leLr e-iz-xzkeh.k lM+d fodkl izkf/kdj.k eq[;ky; Hkksiky dh vksj lwpukFkZ ,oa vko’;d dk;Zogh gsrqA

- izca/kd ¼vkbZ-Vh½ dks osolkbV ij lZdqyj lsD’ku esa viyksM djus gsrq izsf"krA

izeq[k vfHk;ark

e-iz-xzkeh.k lM+d fodkl izkf/kdj.k

Hkksiky